the ESP32 is not an STM32, so STM32CubeMX and Keil don't apply to it

Step 1: Install Arduino IDE and set up ESP32 support

1.1 Download Arduino IDE Go to https://www.arduino.cc/en/software and download Arduino IDE 2.x for your OS. Install it normally.

1.2 Add ESP32 board support The ESP32 is not built into Arduino IDE — you need to add it manually.

- Open Arduino IDE

- Go to File → Preferences

- Find the field labeled "Additional boards manager URLs"

- Paste this URL into it:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK

- Go to Tools → Board → Boards Manager

- In the search box type esp32

- Find the entry called "esp32 by Espressif Systems" — click Install

- Wait for it to finish (it downloads ~200 MB)

1.3 Select your board

- Go to Tools → Board → esp32 → ESP32 Dev Module

  - "ESP32 Core Board V2" is the generic dev board — ESP32 Dev Module is the correct selection for it

- Go to Tools → Port and select the COM port your ESP32 is on

  - On Windows it shows as COM3, COM4, etc.

  - On Mac/Linux it shows as /dev/ttyUSB0 or /dev/tty.SLAB_USBtoUART

  - If you don't see any port, you need the CP2102 USB driver — download from https://www.silabs.com/developers/usb-to-uart-bridge-vcp-drivers

Change upload speed to 115200.

Go to File → Examples → 01.Basics → Blink

1.5 Test

Go to File → Examples → 01.Basics → Blink

Put this code

void setup() {

Serial.begin(115200);

Serial.println("ESP32 is alive!");

}

void loop() {

Serial.println("Running...");

delay(1000);

}

- Click Upload (hold BOOT if needed)

- Open terram term, Set baud rate to 115200

1-)Wire the PIR sensor

VCC  →  ESP32 VIN  (5V pin, not 3.3V)

OUT  →  ESP32 GPIO 23

GND  →  ESP32 GND

Remove previous code

Put ::

#define PIR_PIN 23

void setup() {

Serial.begin(115200);

pinMode(PIR_PIN, INPUT);

Serial.println("Waiting 30 seconds for PIR to warm up...");

Serial.println("Do not move in front of it during this time.");

for (int i = 30; i > 0; i--) {

Serial.print("Ready in: ");

Serial.print(i);

Serial.println(" seconds");

delay(1000);

}

Serial.println("PIR is ready!");

}

void loop() {

int pirState = digitalRead(PIR_PIN);

if (pirState == HIGH) {

Serial.println("MOTION DETECTED — Room: OCCUPIED");

} else {

Serial.println("No motion       — Room: EMPTY");

}

delay(500);

}

And upload.

Now, we use

ASAIR AM2302 for tem/humidity

Pin 1 → VCC  → 3.3V or 5V

Pin 2             → DATA → signal to ESP32 (IO17)

Pin 4 → GND  → ground

Install the DHT library

This time you DO need a library. In Arduino IDE:

- Go to Sketch → Include Library → Manage Libraries

- In the search box type: DHT sensor library

- Find "DHT sensor library by Adafruit" — click Install

- A popup will ask "Install all" — click Install all (this also installs the Adafruit Unified Sensor dependency)

- Wait for it to finish

Use the new code :

#include "DHT.h"

#define DHT_PIN   17

#define DHT_TYPE  DHT22

#define PIR_PIN   23

DHT dht(DHT_PIN, DHT_TYPE);

bool roomOccupied    = false;

bool lastOccupied    = false;

unsigned long lastDHTRead = 0;

void setup() {

Serial.begin(115200);

pinMode(PIR_PIN, INPUT);

dht.begin();

Serial.println("=== Smart Home Presence System ===");

Serial.println("Warming up sensors...");

delay(3000);

Serial.println("System ready.\n");

}

void loop() {

unsigned long now = millis();

// ── PIR: check every loop ──

roomOccupied = (digitalRead(PIR_PIN) == HIGH);

// ── Detect state change (entry/exit event) ──

if (roomOccupied && !lastOccupied) {

Serial.println(">>> SOMEONE ENTERED — turning on lights/atomizer");

// relay control will go here later

}

else if (!roomOccupied && lastOccupied) {

Serial.println("<<< ROOM EMPTY — turning off everything");

// relay control will go here later

}

lastOccupied = roomOccupied;

// ── DHT22: read every 3 seconds ──

if (now - lastDHTRead >= 3000) {

lastDHTRead = now;

float humidity    = dht.readHumidity();

float temperature = dht.readTemperature();

if (!isnan(humidity) && !isnan(temperature)) {

Serial.print("Temp: ");

Serial.print(temperature, 1);

Serial.print(" C  |  Hum: ");

Serial.print(humidity, 1);

Serial.print(" %  |  Room: ");

Serial.println(roomOccupied ? "OCCUPIED" : "EMPTY");

}

}

delay(500);

}

Expected output:

Turn your phone into a local MQTT broker using Termux, and connect everything through your phone's personal hotspot — no internet required.

Base code

#include "DHT.h"

#include <WiFi.h>

#include <PubSubClient.h>

// ── WiFi credentials ──────────────────────────────

const char* ssid     = "";

const char* password = "12345678ok";

// ── MQTT broker ───────────────────────────────────

const char* mqttServer   = "10.130.125.189";

const int   mqttPort     = 1883;

const char* mqttUser     = "admin";

const char* mqttPassword = "01001684495";

// ── MQTT topics ───────────────────────────────────

const char* topicTemp     = "smarthome/room001/temperature";

const char* topicHum      = "smarthome/room001/humidity";

const char* topicRoom     = "smarthome/room001/room";

const char* topicCommand  = "smarthome/room001/command";

// ── Pin definitions ───────────────────────────────

#define DHT_PIN  17

#define DHT_TYPE DHT22

#define PIR_PIN  23

// ── Objects ───────────────────────────────────────

DHT dht(DHT_PIN, DHT_TYPE);

WiFiClient   espClient;

PubSubClient mqtt(espClient);

// ── State ─────────────────────────────────────────

bool roomOccupied  = false;

bool lastOccupied  = false;

unsigned long lastDHTRead    = 0;

unsigned long lastMQTTRetry  = 0;

// ── Called when a command arrives from your phone ──

void onCommandReceived(char* topic, byte* payload, unsigned int length) {

String message = "";

for (int i = 0; i < length; i++) {

message += (char)payload[i];

}

message.trim();

Serial.print("Command received: ");

Serial.println(message);

if (message == "LIGHTS_ON") {

Serial.println(">>> Lights ON command received");

}

else if (message == "LIGHTS_OFF") {

Serial.println(">>> Lights OFF command received");

}

else if (message == "STATUS") {

mqtt.publish(topicRoom, roomOccupied ? "OCCUPIED" : "EMPTY");

}

}

// ── Connect to WiFi ───────────────────────────────

void connectWiFi() {

Serial.print("Connecting to WiFi: ");

Serial.println(ssid);

WiFi.begin(ssid, password);

int attempts = 0;

while (WiFi.status() != WL_CONNECTED) {

delay(500);

Serial.print(".");

attempts++;

if (attempts > 30) {

Serial.println("\nWiFi failed. Check credentials.");

return;

}

}

Serial.println("\nWiFi connected!");

Serial.print("IP address: ");

Serial.println(WiFi.localIP());

}

// ── Connect to MQTT broker ────────────────────────

void connectMQTT() {

mqtt.setServer(mqttServer, mqttPort);

mqtt.setCallback(onCommandReceived);

Serial.print("Connecting to MQTT broker...");

String clientID = "ESP32-" + String(WiFi.macAddress());

if (mqtt.connect(clientID.c_str(), mqttUser, mqttPassword)) {

Serial.println(" connected!");

mqtt.subscribe(topicCommand);

Serial.print("Subscribed to: ");

Serial.println(topicCommand);

} else {

Serial.print(" failed, rc=");

Serial.println(mqtt.state());

}

}

// ── Setup ─────────────────────────────────────────

void setup() {

Serial.begin(115200);

pinMode(PIR_PIN, INPUT);

dht.begin();

Serial.println("=== Smart Home System ===");

connectWiFi();

connectMQTT();

delay(2000);

Serial.println("System ready.");

}

// ── Main loop ─────────────────────────────────────

void loop() {

unsigned long now = millis();

if (!mqtt.connected()) {

if (now - lastMQTTRetry > 5000) {

lastMQTTRetry = now;

Serial.println("MQTT disconnected. Reconnecting...");

connectMQTT();

}

}

mqtt.loop();

// ── PIR check ──

roomOccupied = (digitalRead(PIR_PIN) == HIGH);

if (roomOccupied && !lastOccupied) {

Serial.println(">>> SOMEONE ENTERED");

mqtt.publish(topicRoom, "OCCUPIED");

}

else if (!roomOccupied && lastOccupied) {

Serial.println("<<< ROOM EMPTY");

mqtt.publish(topicRoom, "EMPTY");

}

lastOccupied = roomOccupied;

// ── DHT22: read and publish every 5 seconds ──

if (now - lastDHTRead >= 5000) {

lastDHTRead = now;

float humidity    = dht.readHumidity();

float temperature = dht.readTemperature();

if (!isnan(humidity) && !isnan(temperature)) {

char tempStr[8];

char humStr[8];

dtostrf(temperature, 4, 1, tempStr);

dtostrf(humidity,    4, 1, humStr);

mqtt.publish(topicTemp, tempStr);

mqtt.publish(topicHum,  humStr);

Serial.print("Published → Temp: ");

Serial.print(tempStr);

Serial.print(" C  |  Hum: ");

Serial.print(humStr);

Serial.print(" %  |  Room: ");

Serial.println(roomOccupied ? "OCCUPIED" : "EMPTY");

}

}

delay(500);

}

Step 1 — Install Termux & Mosquitto

Open Termux and run:

bash

pkg update && pkg upgrade

pkg install mosquitto

Step 2 — Create MQTT Password File

bash

mosquitto_passwd -c ~/mosquitto_passwd admin

Enter your password twice when prompted.

Step 3 — Create Mosquitto Config File

bash

nano ~/mosquitto.conf

Paste exactly this:

listener 1883 0.0.0.0

allow_anonymous false

password_file /data/data/com.termux/files/home/mosquitto_passwd

Save with Ctrl+X → Y → Enter.

Step 4 — Enable Phone Hotspot

Go to phone Settings → Personal Hotspot → Turn ON.

Note your hotspot name and password.

Step 5 — Find Hotspot IP

In Termux run:

bash

ifconfig

Look for the ap0 interface:

ap0: inet 10.130.125.189

This is your broker IP. Write it down.

Step 6 — Start Mosquitto Broker

bash

mosquitto -c ~/mosquitto.conf

You should see:

Opening ipv4 listen socket on port 1883

Leave this running. Do not close Termux.

Step 7 — Test the Broker

Open a second Termux session (swipe from left edge) and run:

bash

mosquitto_sub -h 127.0.0.1 -p 1883 -t test -u admin -P YOUR_PASSWORD

Open a third Termux session and run:

bash

mosquitto_pub -h 127.0.0.1 -p 1883 -t test -m "hello" -u admin -P YOUR_PASSWORD

If the word hello appears in the second session → broker is working ✅

Step 8 — Update ESP32 Sketch

Change these lines in your Arduino sketch:

cpp

const char* ssid         = "YourHotspotName";

const char* password     = "YourHotspotPassword";

const char* mqttServer   = "10.130.125.189";  // your ap0 IP

const int   mqttPort     = 1883;

const char* mqttUser     = "admin";

const char* mqttPassword = "YOUR_PASSWORD";

download https://mqtt-explorer.com/

The password is what you set in termux

Now upload the code, and it will start reading